# Oradio Audio System Documentation

## Inhoudsopgave

Oradio Audio System Documentation	1

Inhoudsopgave	1

Overview	1

Key points:	2

Audio Hardware Configuration (config.txt)	2

Purpose	2

Design Rationale	2

ALSA Architecture (asound.conf)	2

Hardware Abstraction	2

Central Mixer (dmix)	2

Why 96 kHz?	2

Resample & Input Bridge	3

Source-Specific Audio Chains	3

MPD Audio Chain	3

Spotify Audio Chain	3

System Sounds Audio Chain	3

MPD Audio Output (mpd.conf)	3

Spotify Audio Output (librespot.service)	3

Module: System Sounds (system_sounds.py)	4

Purpose	4

Design	4

API Reference	4

Stand-alone Test Program	4

Architecture & Interconnections	4

Hardware & Audio Considerations	4

Dependencies	4

Notes & Best Practices	5

## Overview

Oradio’s audio system on Linux (Raspberry Pi) is designed for simultaneous playback of music and system announcements, with independent volume control and optional equalization per source.

The system is based on:

- DigiAMP+ HiFi pcm512x-hifi audio hardware

- ALSA for routing, mixing, and DSP

- MPD for music playback

- Librespot for Spotify playback

- A lightweight Python system sound player for UI and status feedback

All audio sources converge in a single software mixer, ensuring predictable behavior and low latency.

## Key points:

- Centralized ALSA software mixing (dmix)

- Separate ALSA PCM endpoints per audio source

- Independent volume controls for:

  - MPD

  - Spotify

  - System sounds

- Equalizer support per source using alsaequal

- Fire-and-forget system sounds (non-blocking)

- No hardware mixer dependency

- Fail-safe design: system sounds work even if MPD is stopped

# Audio Hardware Configuration (config.txt)

## Purpose

Initializes the audio hardware at kernel level and enables low-latency I²S audio output.

## Design Rationale

- i2s-mmap: Enables memory-mapped I²S for reduced latency and CPU overhead.

- rpi-digiampplus: Activates the HiFiBerry DigiAMP+ as the primary ALSA card.

- unmute_amp: Ensures the amplifier is active immediately after boot, avoiding silent states.

# ALSA Architecture (asound.conf)

## Hardware Abstraction

- Single point of reference for physical audio hardware

- Avoids hard-coded ALSA card indices

## Central Mixer (dmix)

- Mixes all audio sources in software

- Outputs a fixed 96 kHz stereo stream to the DigiAMP+

### Why 96 kHz?

- High-quality internal mixing

- Consistent output format

- All sources are resampled before entering the mixer

## Resample & Input Bridge

- Common entry point for all audio chains

- Centralizes resampling and format normalization

## Source-Specific Audio Chains

Each source has:

- Its own ALSA PCM

- Dedicated volume control

- Optional equalizer stage

## MPD Audio Chain

MPD → equalizer → volume → mixer → DigiAMP+

- Independent volume (VolumeMPD)

- Independent equalizer

- No MPD-internal mixer

## Spotify Audio Chain

Librespot → equalizer → (un)mute →volume → mixer → DigiAMP+

- Two-stage volume control:

  - Hard mute when Spotify is disabled

  - Volume control to adjust relative levels vs other inputs

- Equalizer via alsaequal

## System Sounds Audio Chain

play_sound() → volume →dmixer → DigiAMP+

- Low-latency playback

- No equalizer

- Dedicated volume control

# MPD Audio Output (mpd.conf)

- MPD outputs directly to an ALSA PCM

- All volume handling is delegated to ALSA

- Prevents conflicts between MPD and other sources

# Spotify Audio Output (librespot.service)

- Librespot outputs directly to an ALSA PCM

- All volume handling is delegated to ALSA

- Prevents conflicts between Librespot and other sources

# Module: System Sounds (system_sounds.py)

### Purpose

Plays short system sounds and announcements, such as:

- Startup / shutdown

- Preset changes

- Network status messages

- Error notifications

### Design

- Uses aplay with a dedicated ALSA PCM

- Non-blocking (fire-and-forget)

- No dependency on other audio services

## API Reference

| Method | Description |
| --- | --- |
| play_sound() | Asynchronously send the system sound wav file to SysSound_in |

### Stand-alone Test Program

- Interactive menu

- Stress testing with concurrent sounds

- Randomized playback for robustness testing

# Architecture & Interconnections

- ALSA is the central routing and DSP layer

- All audio sources feed into dmixer

- Volume and equalization are handled per source

- DigiAMP+ receives a single mixed stream

# Hardware & Audio Considerations

- Target platform: Raspberry Pi + HiFiBerry DigiAMP+

- No hardware mixer available

- ALSA dmix is mandatory for concurrent audio

- All PCM names are stable and human-readable

- /etc/alsaequal.bin stores persistent EQ settings

# Dependencies

- Linux ALSA

- MPD

- Python 3.8+

- alsa-utils (aplay)

- alsaequal

- oradio_logging

# Notes & Best Practices

- Do not use hw:X,Y in applications

- Always route audio via named ALSA PCMs

- Keep MPD mixer disabled

- Use system sounds for feedback, not MPD

- Test with multiple concurrent sounds

- Keep asound.conf as the single source of truth

Onderkant formulier