# Audio Hardware

- DAC chip: PCM512x (TI/Burr-Brown family)

  - This is a high-quality DAC used on the Raspberry Pi DigiAMP+ board.

  - Supports 24-bit audio, up to 192 kHz.

  - 2 channels (stereo)

- Driver: pcm512x-hifi-0 (part of the Linux snd-soc SoC audio driver stack)

- Card name: RPi DigiAMP+

- Device: HiFi pcm512x-hifi-0

## About the PCM512x DAC

- TI PCM512x series (like PCM5122, PCM5124)

- Supports:

  - 16–24 bit resolution

  - 44.1–192 kHz sample rates

  - I2S input

  - Integrated voltage reference and filter

- Typical ALSA controls exposed:

  - Digital Playback Volume (hardware master volume)

  - Mute switch

  - Some limited channel mapping

## Volume Control

Command: amixer -c 0 cset name='Digital Playback Volume' <volume>

- volume:

  - Min: 0, -103.5dB (mute)

  - Max: 207, 0dB

# MPD Volume

## Normalization (mpd.conf)

### Replaygain

This setting specifies the type of ReplayGain to use. This setting can have the argument "off", "album", "track" or "auto". "auto" is a special mode that chooses between "track" and "album" depending on the current state of random playback. If random playback is enabled then "track" mode is used.

See <https://wiki.hydrogenaud.io/index.php?title=Replaygain> for more details about ReplayGain.

### replaygain_preamp

This setting sets the pre-amp used for files that have ReplayGain tags.

### replaygain_missing_preamp

This setting sets the pre-amp used for files that do NOT have ReplayGain tags.

### replaygain_limit

This setting enables or disables ReplayGain limiting.

MPD calculates actual amplification based on the ReplayGain tags and replaygain_preamp / replaygain_missing_preamp setting. If replaygain_limit is enabled MPD will never amplify audio signal above its original level. If replaygain_limit is disabled such amplification might occur.

### volume_normalization

This setting enables on-the-fly normalization volume adjustment. This will result in the volume of all playing audio to be adjusted so the output has equal "loudness".

## Volume Control

Simple mixer control 'VolumeMPD',0

Playback channels: Front Left - Front Right

Command: amixer set VolumeMPD <volume>

- volume:

  - Min: 0 | 0% (-51dB)

  - Max: 255 | 100% (0dB)

Used in volume_control.py with fixed value 100%

# Spotify Volume

## Normalization (librespot.service)

### initial-volume

Initial volume in % from 0-100. Default for softvol: 50. For the alsa mixer: the current volume.

### volume-ctrl

Volume control type cubic, fixed, linear, log. Defaults to log.

### enable-volume-normalisation

Enables volume normalisation for librespot

### normalisation-pregain

Pregain (dB) applied by the normalisation. Defaults to 0.

## Mute Control

Simple mixer control VolumeSpotCon1,0

Playback channels: Front Left - Front Right

Command: amixer set VolumeSpotCon1 <volume>

- volume:

  - Min: 0 | 0% (-51dB)

  - Max: 255 | 100% (0dB)

Used in spotify_connect_direct.py with values 0% (mute) and 100% (unmute).

## Volume Control

Simple mixer control VolumeSpotCon2,0

Playback channels: Front Left - Front Right

Command: amixer set VolumeSpotCon2 <volume>

- volume:

  - Min: 0 | 0% (-51dB)

  - Max: 255 | 100% (0dB)

Used in volume_control.py with fixed value 100%

# System Sound Volume

## Normalization

None. Level is determined by levels of system sound wav files.

## Volume Control

Simple mixer control VolumeSysSound,0

Playback channels: Front Left - Front Right

Command: amixer set VolumeSysSound <volume>

- volume:

  - Min: 0 | 0% (-51dB)

  - Max: 255 | 100% (0dB)

Used in volume_control.py with fixed value 90%

# ALSA (asound.conf)

## Normalization

None.

## Equalizer for MPD and Spotify (alsaequal.bin)