# Oradio I²C, Volume, and Backlighting Documentation

## Table of contents

Oradio I²C, Volume, and Backlighting Documentation	1

Table of contents	1

Overview	2

Key points:	2

Module: I2CService	2

Purpose	2

Design & Threading	2

API Reference	2

Error Handling	2

Stand-alone Test Program	2

Module: VolumeControl	3

Purpose	3

Design & Threading	3

ALSA Integration	3

API Reference	3

Stand-alone Test Program	3

Module: Backlighting	3

Purpose	3

Design & Threading	4

Sensor & DAC Handling	4

API Reference	4

Stand-alone Test Program	4

Architecture & Interconnections	4

Data Flow Examples:	4

Hardware & I²C Considerations	5

Dependencies	5

Notes & Best Practices	5

UML Diagrams for Oradio Modules	5

Class Diagram (Textual UML)	5

Sequence Diagram: Volume Change Flow	6

Sequence Diagram: Backlighting Auto-Adjust	7

Thread Lifecycle Diagram	7

I²CService Access Flow	7

## Overview

Oradio’s audio system on Linux (Raspberry Pi) uses three main modules:

- I2CService: Provides thread-safe communication with I²C devices.

- VolumeControl: Reads an analog rotary knob (MCP3021 ADC) and sets ALSA mixer volume.

- Backlighting: Reads ambient light (TSL2591) and controls the MCP4725 DAC for smooth backlight adjustment.

## Key points:

- Both VolumeControl and Backlighting run background threads.

- I2CService is a singleton used by both modules to avoid concurrent bus access.

- All modules include standalone test routines.

# Module: I2CService

## Purpose

- Encapsulates all I²C communication.

- Provides a thread-safe API for reading/writing bytes and blocks.

- Centralizes error handling and logging for I²C operations.

- Supports retries and backoff for write operations.

## Design & Threading

- Singleton: Only one instance exists to prevent multiple threads from accessing SMBus simultaneously.

- Thread Safety: All SMBus operations are guarded by a threading.Lock.

- Retry Logic: Write operations retry I2C_RETRIES times with I2C_BACKOFF seconds delay.

## API Reference

| Method | Description |
| --- | --- |
| read_byte(device, register) | Thread-safe read of a single byte. Returns int or None on failure. |
| write_byte(device, register, value) | Thread-safe write of a single byte with retry/backoff. |
| read_block(device, register, length) | Reads up to 32 bytes. Returns list[int] or None. |
| write_block(device, register, data) | Writes up to 32 bytes with retry/backoff. |

## Error Handling

- All errors are logged using oradio_log.error or .warning.

- Exceeding block size logs an error immediately.

- Failed writes after all retries are logged as errors.

## Stand-alone Test Program

- Scans /dev/i2c-* to find available buses.

- Probes expected devices (MCP3021, TSL2591, MCP4725, HUSB238).

- Reports missing and unexpected devices using color-coded console output.

# Module: VolumeControl

## Purpose

- Reads analog volume knob via MCP3021 ADC over I²C.

- Updates ALSA mixer volume in raw units.

- Sends asynchronous notifications of significant volume changes.

## Design & Threading

- Background Thread: _volume_manager polls ADC continuously.

- Adaptive Polling:

  - Fast polling (POLLING_MIN_INTERVAL) when knob changes.

  - Slow polling (POLLING_MAX_INTERVAL) when idle.

  - Step size POLLING_STEP for adaptive intervals.

- Notification Queue: Sends messages once per significant change.

## ALSA Integration

- Uses AlsaVolume wrapper class.

- Avoids redundant ALSA calls by caching the last set volume.

- Ensures volume is clamped to [VOLUME_MINIMUM..VOLUME_MAXIMUM].

## API Reference

| Method | Description |
| --- | --- |
| start() | Start the volume manager thread. |
| stop() | Stop the volume manager thread safely. |
| set_notify() | Re-arm the notification queue trigger. |

## Stand-alone Test Program

- Interactive menu with options to start/stop the thread and set notifications.

- Queue listener process prints received volume change messages.

# Module: Backlighting

## Purpose

- Automatically adjusts display backlight using ambient light readings.

- Supports manual override: off or maximum brightness.

- Smooth transitions prevent sudden brightness jumps.

## Design & Threading

- Background Thread: _backlight_manager continuously adjusts DAC output.

- Smooth Transition Algorithm: Gradually moves current backlight value toward target over TRANSITION_TIME with updates every ADJUST_INTERVAL.

- Thresholding: DAC writes occur only if change ≥ CHANGE_THRESHOLD.

## Sensor & DAC Handling

- TSL2591 ALS sensor: Reads visible light (16-bit) over I²C.

- MCP4725 DAC: Writes 12-bit output via I²C.

  - SAVE_PERSISTENT for EEPROM storage (boot-safe).

  - SAVE_VOLATILE for fast, non-persistent updates.

## API Reference

| Method | Description |
| --- | --- |
| start() | Start auto-adjust thread. |
| stop() | Stop auto-adjust thread. |
| off() | Turn off backlight and stop auto-adjust. |
| maximum() | Set backlight to maximum and stop auto-adjust. |
| read_sensor() | Return (raw_visible_light, lux, interpolated DAC value) for monitoring. |

## Stand-alone Test Program

- Interactive menu to start/stop auto-adjust, set off/maximum, and monitor sensor values in real time.

# Architecture & Interconnections

- I2CService is the central communication hub, shared by both VolumeControl and Backlighting.

- VolumeControl → I2CService: Reads MCP3021 ADC (0x4D).

- Backlighting → I2CService:

  - Reads TSL2591 ALS sensor (0x29).

  - Writes MCP4725 DAC (0x60).

- Thread Safety: Single Lock in I2CService ensures no collisions.

- Queues & Notifications: Volume changes are reported asynchronously to calling code.

# Data Flow Examples:

- Volume change:
Knob → MCP3021 → I2CService → VolumeControl → ALSA Mixer → Notification Queue

- Backlight auto-adjust:
Ambient light → TSL2591 → I2CService → Backlighting → Interpolation → MCP4725 DAC → Backlight

# Hardware & I²C Considerations

- Supports Raspberry Pi (bus 1).

- Maximum block read/write: 32 bytes (SMBus limitation).

- Expected devices:

  - MCP3021 ADC (0x4D)

  - TSL2591 ALS sensor (0x29)

  - MCP4725 DAC (0x60)

  - HUSB238 USB-C Power Controller (0x08)

- Ensure /dev/i2c-1 permissions allow access (sudo may be required).

# Dependencies

- Python 3.8+

- smbus2 for I²C

- alsaaudio for ALSA integration

- oradio_logging for logging

- singleton decorator

- oradio_utils for safe queue operations

# Notes & Best Practices

- Thread Lifecycle: Always start threads via start() and stop with stop() before program exit.

- Singleton Usage: Only one instance of I2CService ensures safe multi-threaded access.

- Error Handling: All I²C and ALSA errors are logged; modules attempt retries or continue gracefully.

- Adaptive Polling & Smoothing: Optimized to reduce CPU usage while maintaining responsive behavior.

- Stand-alone Testing: Each module has an interactive menu for diagnostics, ideal during development or deployment.

# UML Diagrams for Oradio Modules

- Class relationships (I2CService singleton, VolumeControl, Backlighting, AlsaVolume).

- Sequence flows for volume adjustment and backlight auto-adjust.

- Thread lifecycle management.

- I²C access flow, showing safe communication with hardware.

Bovenkant formulier

## Class Diagram (Textual UML)

+---------------------------+

|       I2CService          |<<singleton>>

|---------------------------|

| - _bus: SMBus             |

| - _lock: Lock             |

|---------------------------|

| + read_byte(dev, reg)     |

| + write_byte(dev, reg, v) |

| + read_block(dev, reg, l) |

| + write_block(dev, reg, d)|

+---------------------------+

^                      ^

|                      |

+----------------------------+   +----------------------------+

|     VolumeControl          |   |       Backlighting         |

|----------------------------|   |----------------------------|

| - _i2c_service: I2CService |   | - _i2c_service: I2CService |

| - _alsa: AlsaVolume        |   | - _thread: Thread          |

| - _queue: Queue            |   | - _running: Event          |

| - _thread: Thread          |   |----------------------------|

| - _running: Event          |   | + start()                  |

| - _armed: bool             |   | + stop()                   |

|----------------------------|   | + off()                    |

| + start()                  |   | + maximum()                |

| + stop()                   |   | + read_sensor() -> tuple   |

| + set_notify()             |   +----------------------------+

+----------------------------+

|

v

+-------------------------+

|       AlsaVolume        |

|-------------------------|

| - _mixer: Mixer         |

| - _last_set_raw: int    |

|-------------------------|

| + set(raw_value)        |

+-------------------------+

Notes:

- Both VolumeControl and Backlighting depend on the singleton I2CService.

- VolumeControl composes an AlsaVolume object for ALSA mixer control.

- Threads in both modules allow continuous polling and auto-adjustment.

## Sequence Diagram: Volume Change Flow

Volume Knob (ADC) -> I2CService: read_block()

I2CService -> MCP3021: read ADC data

I2CService --> VolumeControl: return ADC value

VolumeControl -> AlsaVolume: set(volume)

AlsaVolume --> ALSA Mixer: setvolume()

VolumeControl -> Queue: send volume changed message

Notes:

- VolumeControl polls the ADC in _volume_manager.

- Adaptive polling: fast when knob moves, slow when idle.

- Messages sent only once per significant change via the queue.

## Sequence Diagram: Backlighting Auto-Adjust

Backlighting._backlight_manager -> I2CService: read_byte(VISIBLE_LIGHT_LOW/HIGH)

I2CService -> TSL2591: return sensor data

I2CService --> Backlighting: raw visible light

Backlighting -> _interpolate_backlight: compute DAC target

Backlighting -> I2CService: write_block(DAC value)

I2CService -> MCP4725: set DAC output

Notes:

- Smooth transition: backlight value gradually moves toward target.

- Writes occur only if change exceeds CHANGE_THRESHOLD.

- Thread runs in the background (_backlight_manager) with ADJUST_INTERVAL updates.

## Thread Lifecycle Diagram

VolumeControl / Backlighting

-----------------------------

start() -------------------> creates Thread(target=_manager)

|

v

_manager loop:

poll sensor/ADC

calculate target

update ALSA/DAC

sleep(interval)

stop() --------------------> _running.clear()

thread joins

cleanup

Notes:

- Both modules implement safe thread start/stop.

- Threads are daemon, so they won’t block program exit.

- _running Event signals threads to stop.

## I²CService Access Flow

VolumeControl / Backlighting

|

v

I2CService (singleton)

|

+-- read_byte/write_byte/write_block/read_block

|

v

Physical I²C device (ADC, ALS sensor, DAC)

Notes:

- Ensures mutual exclusion via _lock.

- Thread-safe access prevents bus collisions.

- Logging included for all operations.

Onderkant formulier