Inhoudsopgave

Introduction	1

Section: Normal flow	2

Power on	2

On-button pressed	2

Previous: Oradio Off	2

Previous: Oradio playing a preset playlist	2

Previous: Oradio On	2

Long-press detected	2

Preset-button pressed	2

Previous: Oradio Off, On, or playing other preset playlist	2

Previous: Oradio playing same preset playlist with songs on USB	3

Previous: Oradio playing same preset playlist is webradio	3

Volume-knob rotated	3

Previous: Oradio Off	3

Off-button pressed	3

Previous: Access point inactive	3

Previous: Access point active	3

Section: Access point flow	3

Access point stopped	3

Access point started	3

Section: Error flow	4

USB removed	4

USB inserted	4

Wifi disconnected	4

Previous: Oradio not Off	4

Previous: Playing playlist webradio	4

Wifi connected	4

Previous: Oradio not Off	4

# Introduction

Sections co-exist and run independent from other sections.

Each section describes a series of actions i.e. steps for a specific action.

Each step can be interrupted by an error triggering the error flow.

# Section: Normal flow

## Power on

When RPI boots the Off-LED is on, other LEDs and backlight is off.

- Initialize HW (Power, LEDs, Backlight, Buttons, Volume, USB, wifi)

- Set Off-LED blink, other LEDs off

- Wait for USB status

- Initialize MPD

- Play startup tune

- Set Off-LED off, other LEDs off

## On-button pressed

- Play click sound

- Set On-LED on, other LEDs off

### Previous: Oradio Off

- Play On tune

#### If last played list is empty

- Play preset1 playlist

#### If last played list is not empty

- Play last playlist

### Previous: Oradio playing a preset playlist

- Play On tune

### Previous: Oradio On

#### If playlist songs are on USB

- Play next-song message

- Play next song from playlist

#### If playlist is webradio

- Play On tune

### Long-press detected

- Set On-LED blink, all other LEDs off

- Start access point

## Preset-button pressed

Legend: # can be 1, 2, or 3

- Play click sound

### Previous: Oradio Off, On, or playing other preset playlist

- Set preset#-LED on, other LEDs off

- Play preset# message

- Play preset# playlist

### Previous: Oradio playing same preset playlist with songs on USB

- Play next-song message

- Play next song from playlist

### Previous: Oradio playing same preset playlist is webradio

- Play preset# message

## Volume-knob rotated

- Set volume to knob position

### Previous: Oradio Off

- Set On-LED on, other LEDs off

#### If last played list is empty

- Play preset1 playlist

#### If last played list is not empty

- Play last playlist

## Off-button pressed

- Play click sound

### Previous: Access point inactive

- Set Off-LED on, other LEDs off

- Play shutdown tune

- Set Off-LED off

### Previous: Access point active

- Set all LEDs off

# Section: Access point flow

## Access point stopped

- Play AP-stopped message

## Access point started

- Play AP-started message

TODO: Add actions for web interface pages

# Section: Event flow

## USB removed

- Stop playing playlist

- Set Off-LED blink, other LEDs off

- Play USB absent message

## USB inserted

- Play USB present message

- Set all LEDs off

## Wifi disconnected

### Previous: Oradio not Off

- Play connected message

### Previous: Playing playlist webradio

- Stop playing playlist

## Wifi connected

### Previous: Oradio not Off

- Play connected message

TODO: Add other module events

# Section: Error flow

TODO: Add errors and actions