# Raspberry Pi Throttling Monitor

## Overview

The RPi Throttling Monitor is a singleton-based background service that continuously monitors the throttling status of a Raspberry Pi using the vcgencmd get_throttled command.

The monitor detects:

- Under-voltage conditions

- ARM CPU frequency capping

- Active CPU throttling

- Thermal soft-limit conditions

It also reports historical throttling events that have occurred since the last reboot.

Whenever a throttling condition is detected, the monitor:

- Writes a warning message to the Oradio logging subsystem.

- Publishes an error message to the Oradio messaging framework.

- Tracks state transitions to prevent duplicate log entries.

The monitor starts automatically when the module is imported.

# Architecture

The module implements a singleton class:

RPiThrottlingMonitor

A single instance is automatically created:

throttling_monitor = RPiThrottlingMonitor()

The monitor runs in a dedicated daemon thread and periodically polls the Raspberry Pi firmware for throttling information.

# Functional Design

## Startup Sequence

When the module is imported:

- The singleton instance is created.

- Historical throttling flags are read.

- Historical events are reported if present.

- A background monitoring thread is started.

Initialization flow:

Module Import
      │
      ▼
Create Singleton
      │
      ▼
Read get_throttled Status
      │
      ├── Historical Events Found
      │        │
      │        ├── Log Warning
      │        └── Publish Error
      │
      ▼
Start Monitoring Thread

# Throttling Flag Definitions

The Raspberry Pi firmware returns a hexadecimal bitmask.

Example:

throttled=0x50005

The monitor interprets the following bits.

## Active Status Flags

| Bit | Hex Value | Description |
| --- | --- | --- |
| 0 | 0x1 | Under-voltage detected |
| 1 | 0x2 | ARM frequency capped |
| 2 | 0x4 | Currently throttled |
| 3 | 0x8 | Soft temperature limit active |

These flags represent the current hardware state.

## Historical Status Flags

| Bit | Hex Value | Description |
| --- | --- | --- |
| 16 | 0x10000 | Under-voltage has occurred |
| 17 | 0x20000 | ARM frequency capping has occurred |
| 18 | 0x40000 | Throttling has occurred |
| 19 | 0x80000 | Soft temperature limit has occurred |

These flags remain set until the Raspberry Pi is rebooted.

# Polling Mechanism

The monitor executes:

vcgencmd get_throttled

at a configurable interval.

Default interval:

1.0 second

Only active-state bits are monitored for changes:

ACTIVE_MASK = 0x1 | 0x2 | 0x4 | 0x8

Historical bits are ignored during normal polling to avoid repeated notifications.

# State Transition Detection

The monitor maintains a cached copy of the previously observed active state:

self._last_active_flags

The current state is compared with the previous state on every polling cycle.

## Entering a Throttled State

When active flags change from:

0x0 → non-zero

or

one condition → different condition

the monitor:

- Decodes active flags.

- Logs a warning.

- Publishes an error message.

Example:

RPi throttling ENTERED: Under-voltage detected

## Leaving a Throttled State

When active flags change to:

0x0

the monitor logs:

RPi throttling CLEARED

No error message is published when the condition clears.

# Threading Model

The monitoring loop runs inside a daemon thread.

Thread properties:

Thread(
    target=self._run,
    name="rpi-throttling-monitor",
    daemon=True
)

Characteristics:

- Automatically terminates when the application exits.

- Can be interrupted immediately through an Event object.

- Supports clean shutdown.

# Error Reporting

The monitor integrates with the Oradio messaging framework.

## Error Publication

When throttling is detected:

publish_error(
    ErrorMessage(
        MESSAGE_THROTTLING_SOURCE,
        MESSAGE_THROTTLING_ERROR_THROTTLED
    )
)

This allows other subsystems to react to hardware health problems.

# Logging

The monitor uses the Oradio logging subsystem.

Typical log entries:

RPi HEALTH WARNING (since boot): Under-voltage has occurred

RPi throttling ENTERED: Currently throttled

RPi throttling ENTERED: Under-voltage detected, Currently throttled

RPi throttling CLEARED

# Test Mode

The monitor includes an integrated test mode for validation without requiring actual hardware faults.

## Enable Test Mode

throttling_monitor.enable_test_mode()

In test mode, firmware queries are bypassed.

## Simulate Throttling

Example:

throttling_monitor.force_throttled_test(0x4)

Produces:

Currently throttled

### Under-voltage

force_throttled_test(0x1)

### Frequency Capped

force_throttled_test(0x2)

### Thermal Throttling

force_throttled_test(0x4)

### Temperature Soft Limit

force_throttled_test(0x8)

### Multiple Conditions

force_throttled_test(
    0x1 | 0x2 | 0x4 | 0x8
)

## Clear Test Condition

throttling_monitor.clear_throttled_test()

Result:

RPi throttling CLEARED

## Disable Test Mode

throttling_monitor.disable_test_mode()

Restores normal hardware polling.

# Public Interface

## Constructor

RPiThrottlingMonitor(interval=1.0)

Creates the singleton monitor and starts monitoring.

## start()

start()

Starts the monitoring thread if not already running.

## stop()

stop()

Signals the monitoring thread to terminate.

## enable_test_mode()

enable_test_mode()

Switches the monitor to simulated operation.

## disable_test_mode()

disable_test_mode()

Returns to normal hardware monitoring.

## force_throttled_test()

force_throttled_test(flags)

Injects a simulated throttling state.

## clear_throttled_test()

clear_throttled_test()

Clears all simulated throttling conditions.

# Design Considerations

## Advantages

- Automatic startup.

- Singleton implementation prevents duplicate monitors.

- Low CPU overhead.

- Immediate detection of hardware issues.

- Historical fault detection at startup.

- Built-in test framework.

- Clean integration with Oradio logging and messaging systems.

## Limitations

- Requires Raspberry Pi firmware utility:

vcgencmd

- Historical flags cannot be cleared without rebooting.

- Polling interval determines detection latency.

- Test mode is intended for diagnostics and should not be enabled in production.

# Typical Usage

The monitor is self-starting.

import throttling_monitor

No further initialization is required.

Monitoring begins automatically and continues until application shutdown.