# USB Monitoring and WiFi Import Service

## Overview

The USB Monitoring Service provides automatic detection of USB insertion and removal events for Oradio systems and optionally imports WiFi credentials from a configuration file stored on a designated USB drive.

The service uses the Linux filesystem event framework through the watchdog library to monitor a marker file that represents the presence of an auto-mounted USB device.

Key capabilities include:

- Detection of USB insertion and removal

- Publication of USB state changes via the Oradio messaging framework

- Automatic import of WiFi credentials from USB media

- Validation of imported WiFi configuration data

- Integration with NetworkManager

- Error reporting through the Oradio messaging subsystem

- Singleton event handling to prevent duplicate registrations

# Purpose

The service enables non-technical users to:

- Insert an ORADIO USB drive.

- Automatically configure WiFi networks.

- Remove the USB drive.

- Allow the system to connect using the imported credentials.

The service eliminates the need for direct command-line or graphical configuration of wireless networks.

# Architecture

The service consists of two primary classes:

USBService
    │
    ▼
Watchdog Observer
    │
    ▼
USBObserver (Singleton)
    │
    ├── USB State Detection
    ├── WiFi Configuration Import
    ├── Message Publishing
    └── Error Reporting

# Component Overview

## USBObserver

The USBObserver class inherits from:

FileSystemEventHandler

and is decorated with:

@singleton

This ensures that only one observer instance exists within a process.

Responsibilities:

- Detect USB insertion

- Detect USB removal

- Publish USB state changes

- Import WiFi credentials

- Validate configuration files

- Report configuration errors

## USBService

The USBService class provides the high-level interface to the monitoring subsystem.

Responsibilities:

- Create watchdog observer

- Register USBObserver

- Start observer thread

- Verify observer startup

- Provide current USB state queries

# USB Detection Mechanism

The operating system automatically mounts USB drives labeled:

ORADIO

A marker file is created when the USB device is mounted:

/run/usb_present

and removed when the device is disconnected.

The service monitors:

/run

for filesystem events.

# Filesystem Layout

## USB Mount Point

<USB_MOUNT_POINT>

Example:

/media/ORADIO

## USB Presence Marker

/run/usb_present

Purpose:

- Created when USB is mounted

- Removed when USB is unmounted

The watchdog observer uses creation and deletion events on this file to determine USB state changes.

## WiFi Configuration File

Expected location:

<USB_MOUNT_POINT>/Wifi_invoer.json

Example:

/media/ORADIO/Wifi_invoer.json

# Startup Sequence

When USBService starts:

Create Observer
        │
        ▼
Create USBObserver
        │
        ▼
Check Existing Mount State
        │
        ├── Mounted
        │      │
        │      ├── Publish USB_PRESENT
        │      └── Import WiFi Networks
        │
        └── Not Mounted
               │
               └── Publish USB_ABSENT
        │
        ▼
Register Watchdog Handler
        │
        ▼
Start Observer Thread

# USB State Detection

## USB Present

When the marker file is created:

/run/usb_present

the observer publishes:

CommandMessage(
    MESSAGE_USB_SOURCE,
    STATE_USB_PRESENT
)

and logs:

USB inserted

## USB Removed

When the marker file is deleted:

/run/usb_present

the observer publishes:

CommandMessage(
    MESSAGE_USB_SOURCE,
    STATE_USB_ABSENT
)

and logs:

USB removed

# Messaging Integration

The service communicates through the Oradio publish-subscribe framework.

## Command Messages

Published events:

| Event | Message |
| --- | --- |
| USB inserted | STATE_USB_PRESENT |
| USB removed | STATE_USB_ABSENT |

## Error Messages

Published when:

- JSON parsing fails

- WiFi file cannot be read

- Invalid WiFi configuration detected

- NetworkManager registration fails

- Credentials file cannot be deleted

- Watchdog observer startup fails

Error source:

MESSAGE_USB_SOURCE

# WiFi Configuration Import

When a USB drive is detected, the service checks for:

Wifi_invoer.json

If present, the file is read and processed.

# JSON Format

The expected structure is:

{
  "networks": [
    {
      "SSID": "HomeWiFi",
      "PASSWORD": "secret123"
    },
    {
      "SSID": "GuestNetwork",
      "PASSWORD": ""
    }
  ]
}

# Validation Rules

Each network entry must be a JSON object.

Required fields:

| Field | Required |
| --- | --- |
| SSID | Yes |
| PASSWORD | Yes |

## SSID Validation

Requirements:

- Must be a string

- Cannot be empty

- Cannot contain only whitespace

- Maximum length: 32 characters

Examples:

Valid:

{
  "SSID": "OfficeWiFi"
}

Invalid:

{
  "SSID": ""
}

## Password Validation

Requirements:

- Must be a string

- Empty string allowed (open network)

- Non-empty passwords must contain at least 8 characters

Valid:

{
  "PASSWORD": ""
}

{
  "PASSWORD": "password123"
}

Invalid:

{
  "PASSWORD": "12345"
}

# Network Registration

Validated networks are registered through:

networkmanager_add(ssid, password)

Successful registration:

Network 'MyWiFi' added to NetworkManager

Failed registration:

Failed to add 'MyWiFi' to NetworkManager

and an error message is published.

# File Cleanup

After successful import of all networks:

Wifi_invoer.json

is deleted.

Purpose:

- Prevent duplicate imports

- Remove sensitive credentials from removable media

Process:

Import Networks
        │
        ▼
   All Valid?
        │
   ┌────┴────┐
   │         │
  Yes       No
   │         │
Delete     Keep
File       File

# Error Handling

The service reports errors but continues operating.

Errors include:

| Condition | Action |
| --- | --- |
| JSON syntax error | Publish error |
| Missing networks list | Publish error |
| Invalid SSID | Publish error |
| Invalid password | Publish error |
| NetworkManager failure | Publish error |
| File deletion failure | Publish error |
| Observer startup failure | Publish error |

# Current State Query

Applications may query current USB state directly:

state = monitor.get_state()

Returns:

STATE_USB_PRESENT

or

STATE_USB_ABSENT

The result is obtained from a direct mount-point check and does not rely on cached state.

# Threading Model

The service uses the watchdog observer thread.

Main Thread
     │
     ▼
USBService
     │
     ▼
Watchdog Observer Thread
     │
     ▼
USBObserver Event Callbacks

Benefits:

- Non-blocking operation

- Low CPU usage

- Immediate filesystem event detection

# Stand-Alone Test Mode

Executing the module directly starts an interactive test application.

python usb_service.py

Available tests:

| Option | Function |
| --- | --- |
| 1 | Get USB state |
| 2 | Simulate USB insertion |
| 3 | Simulate USB removal |

The test environment also subscribes to:

- COMMAND messages

- ERROR messages

and prints all received events.

# Typical Usage

## Start Monitoring

usb_service = USBService()

Monitoring begins immediately.

## Query State

state = usb_service.get_state()

## Example Subscriber

queue = subscribe_commands()

while True:
    message = safe_get(queue)

    if message.message == STATE_USB_PRESENT:
        print("USB inserted")

    elif message.message == STATE_USB_ABSENT:
        print("USB removed")

# Design Advantages

- Event-driven architecture

- No polling required

- Automatic WiFi provisioning

- Singleton observer protection

- Integration with Oradio messaging

- Low resource consumption

- Immediate detection of filesystem changes

- Automatic cleanup of imported credentials

# Limitations

- Requires Linux auto-mount support

- Depends on watchdog package

- Requires USB device label configuration

- Assumes presence marker file management by the operating system

- Supports only the predefined WiFi JSON format

- NetworkManager must be available