# I2C Class Diagram (Textual UML)

+----------------------------------+

|           VolumeControl          |

|----------------------------------|

| + start()                        |

| + stop()                         |

| + arm()                          |

| - _volume_manager()              |

| - _read_adc()                    |

| - _set_volume(adc_value)         |

+----------------------------------+

|					^(singleton)

|					|

+----------------------+      +----------------------------------------+

|    AlsaVolume        |      |           I2CService                   |

|----------------------|      |----------------------------------------|

| + set(raw_value)     |      | + write_byte(device, register, value)  |

+----------------------+      | + read_byte(device, register)          |

| + write_block(device, register, data)  |

| + read_block(device, register, length) |

+----------------------------------------+

+-----------------------------------+

|          Backlighting             |

|-----------------------------------|

| + start()                         |

| + stop()                          |

| + off()                           |

| + maximum()                       |

| + read_sensor() -> tuple          |

| - _backlight_manager()            |

| - _write_dac(value, eeprom=False) |

| - _initialize_sensor()            |

| - _read_visible_light()           |

| - _interpolate_backlight(als_val) |

+-----------------------------------+

^ (singleton)

|

+----------------------------------------+

|           I2CService                   |

|----------------------------------------|

| + write_byte(device, register, value)  |

| + read_byte(device, register)          |

| + write_block(device, register, data)  |

| + read_block(device, register, length) |

+----------------------------------------+

## Relationships

- Singleton: I2CService is a singleton. Both VolumeControl and Backlighting use it for I²C operations.

- Composition: VolumeControl contains an instance of AlsaVolume for ALSA mixer operations.

- Threaded Operations: Both VolumeControl and Backlighting run background threads for continuous operation.

# Module: I2CService – Oradio I2C Access Module

## Summary:

The I2CService module provides thread-safe access to I2C devices on a Raspberry Pi or similar Linux-based systems. It is designed to manage I2C communication safely across multiple threads, without implementing Packet Error Checking (PEC).

## Key Features:

- Thread-safe I2C operations using a Lock to prevent concurrent bus access.

- Read and write operations for bytes, words, and blocks (up to 32 bytes per SMBus limitations).

- Error logging for all I2C operations via the oradio_log module.

- Supports detection and probing of connected I2C devices.

- Includes a stand-alone test program to scan all I2C buses and report missing or unexpected devices.

## Supported Devices (configured by default):

| Address | Device Name |
| --- | --- |
| 0x4D | MCP3021 - A/D Converter |
| 0x29 | TSL2591 - Ambient Light Sensor |
| 0x60 | MCP4725 - D/A Converter |
| 0x08 | HUSB238 - USB-C Power Controller |

## Dependencies:

- Python 3.8+

- smbus2 (I2C communication)

- oradio_logging (custom logging)

- singleton (singleton decorator)

## Notes:

- Maximum block read/write is 32 bytes (SMBus limitation).

- The module handles permissions and ensure /dev/i2c-* devices are accessible.

- Provides a stand-alone test routine to probe buses and devices with color-coded console output.

## References:

- smbus2 GitHub Repository

# Module: VolumeControl – Oradio Volume Control Module

## Summary:

The VolumeControl module provides a robust interface to manage audio volume via an analog rotary knob (MCP3021 ADC) and ALSA mixer on Linux systems (e.g., Raspberry Pi). It continuously monitors the ADC for changes, scales the reading to ALSA raw volume units, updates the mixer, and triggers asynchronous notifications of significant changes. Threading is used for continuous polling while allowing safe start/stop operations.

## Key Features:

- Analog volume knob support: Reads 10-bit values from MCP3021 ADC via I²C.

- ALSA integration: Sets mixer volume in raw units using alsaaudio.Mixer.

- Threaded operation: Volume management runs in a background thread with adaptive polling intervals.

- Event notifications: Sends messages on significant volume changes through a queue.

- Safety and efficiency: Avoids redundant ALSA calls, clamps volume within configured min/max, and logs all operations/errors.

- Standalone test program: Interactive menu for testing volume control and monitoring queue messages.

## Constants & Configuration:

- Raw volume range: 105–215 (ALSA units)

- ADC polling: Adaptive interval 0.05–0.3 seconds, step 0.01

- ADC tolerance: 5 units (changes smaller than this are ignored)

- MCP3021 I²C address: 0x4D, data register: 0x00

- ALSA mixer name: "Digital" (default)

## Dependencies:

- Python 3.8+

- alsaaudio (ALSA C extension)

- oradio_logging (custom logging)

- i2c_service (I²C communication wrapper)

- oradio_utils (safe queue operations)

## Notes:

- Adaptive polling ensures quick response when the knob is moved, and slower idle polling to reduce CPU usage.

- The module gracefully handles ALSA errors and missing ADC readings.

- Designed for development; ensure ALSA mixer and /dev/i2c-* devices are accessible.

## References:

- ALSA Python module: pyalsaaudio documentation

- MCP3021 datasheet (for ADC volume knob)

# Module: Backlighting – Oradio Backlighting Control Module

## Summary:

The Backlighting module provides automatic control of the Oradio backlight using a TSL2591 ambient light sensor and an MCP4725 DAC. It continuously monitors ambient light and adjusts the backlighting level smoothly, starting with a low initial backlight level at boot. The module supports manual overrides, on/off, and maximum backlight settings, and runs auto-adjustment in a background thread for continuous operation.

## Key Features:

- Automatic ambient light adjustment: Maps TSL2591 sensor readings to backlight brightness.

- Smooth transitions: Gradually changes DAC output to avoid sudden brightness jumps.

- Manual overrides: Supports turning the backlight off or to maximum, stopping auto-adjust.

- Threaded operation: Auto-adjust runs in a background thread with adjustable intervals.

- Safe DAC writes: Minimizes EEPROM writes to reduce wear and preserves hardware longevity.

- Standalone test program: Interactive menu to test backlight control, sensor readings, and manual functions.

## Constants & Configuration:

- Ambient light sensor (TSL2591) address: 0x29

- DAC (MCP4725) address: 0x60

- Backlight levels: OFF, MIN, MID, MAX

- Ambient light thresholds: ALS_MIN, ALS_MID, ALS_MAX

- Transition time: 10 seconds for full backlight change

- Update interval: 0.5 seconds

- DAC change threshold: 5 units (to trigger write)

- Thread timeout: 3 seconds

## Dependencies:

- Python 3.8+

- i2c_service (I²C communication wrapper)

- oradio_logging (custom logging)

## Notes:

- Adaptive smoothing ensures smooth brightness transitions.

- Ensures initial backlight is off at startup to prevent high brightness.

- Designed for development and testing; requires I²C access to TSL2591 and MCP4725.

- Supports interactive test menu for standalone execution.

## References:

- MCP4725 DAC datasheet

- TSL2591 ALS sensor datasheet